# Features

Mode Indicator

3-1 Switch Adapter

USB Connector

Switch Input Ports

Reset Button

Mode Indicator

USB Connector

Switch Input Jack

Reset Button

Operating Mode in Single Switch Mode

| Input |  | Output |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Activation | Input Switch | Mode 1 (Teal) | Mode 2 (Purple) | Mode 3 (Yellow) | Mode 4 (Green) | Mode 5 (Orange) |
| One Switch | Switch 1 | Enter | F1 | Left Click | F13 | Play/Pause |

Operating Mode in Multiple Switch Mode

| Input |  | Output |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Activation | Input Switch | Mode 1 (Teal) | Mode 2 (Purple) | Mode 3 (Yellow) | Mode 4 (Green) | Mode 5 (Orange) |
| One Switch | Switch 1 | Enter | F1 | Left Click | F13 | Play/Pause |
|  | Switch 2 | Space | F2 | Right Click | F14 | Volume Up |
|  | Switch 3 | Tab | F3 | Middle Click | F15 | Volume Down |
| Two Switch | Switch 1+2 | Backspace | F4 | Left Click | F16 | Next Track |
|  | Switch 2+3 | Escape | F5 | Right Click | F17 | Previous Track |
|  | Switch 1+3 | Enter | F1 | Left Click | F13 | Play/Pause |
| Three Switch | Switch 1+2+3 | Enter | F1 | Left Click | F13 | Play/Pause |

The operating mode can be changed by performing a long press (holding for longer than four seconds) on any connected switch.

## Setup

### Single Switch Configuration

### 1-3 Switches

If the connected device does not have a USB-A port, an adapter will be needed. If used with an iOS device, it will need a separate power source as described in the iOS use section.

# Further Information

| If you are looking for information on troubleshooting, support, or any other questions, refer to the User Guide. | User Guide |
| --- | --- |